Part 1 — The Real Numbers (Why This Project Matters) STATS

Here are actual, sourced statistics:

How crowded is it right now?

- As of October 1, 2025, there are 13,026 active satellites — a 23% year-on-year increase. Just six years ago, fewer than 2,000 were operational.

- As of 2026, tracking data shows 18,049 active satellites in orbit. More than 70% of all satellites ever launched were launched after 2019.

- As of April 2026, cataloged objects exceed 40,000. Smaller but dangerous objects amount to more than 1 million debris fragments greater than 1 cm — largely undetected.

How bad is the collision problem?

- The United States Space Force 19th Space Defense Squadron has generated approximately 600,000 Conjunction Data Messages (CDMs) per day — as of 2023. Compared to 2020, the daily rate tripled.

- One company estimated that conjunctions where satellites came within 1 km of each other increased from 2,000 per month in 2017 to 4,000 per month in 2020. In just the six-month period between December 2022 and May 2023, SpaceX Starlink satellites alone maneuvered more than 25,000 times.

- In 2025, Starlink satellites fired their thrusters to dodge collisions roughly 300,000 times — about 822 times a day — up about 50% from 200,000 in 2024. At the current rate, SpaceX could be performing close to a million maneuvers annually by 2027.

The ESA email story — why current process fails:

- In a real 2019 event, ESA and SpaceX coordinated a collision avoidance maneuver over email. ESA pinged SpaceX when risk reached 1-in-10,000, but heard nothing back — a bug in SpaceX's on-call paging system stopped them from seeing the rising risk estimates. ESA maneuvered alone. This is what real coordination looks like today. Email. Paging systems that crash.

Where it's going:

- The number of satellites in orbit is expected to exceed 100,000 by 2030, making collision avoidance activities of utmost importance.

- SpaceX's Starlink alone has 8,366 satellites — adding 1,963 in 2024 alone. China grew its fleet 25% in one year.

The Kessler Syndrome — the worst-case:

- Models predict that business-as-usual scenarios could trigger irreversible orbital collapse within 250 years. Some orbital shells already exceed critical debris density thresholds. The 2009 Iridium–Cosmos collision alone created over 2,300 trackable fragments.

Part 2 — "Won't They Have Already Solved It By the Time We Finish?"

This is a genuinely smart concern. Here's the honest answer:

They are actively trying to solve it — and that's actually good for your project, not bad.

The space industry solving this problem means there's a real market and urgency for research. Your project is not claiming to deploy satellites. It's a research protocol — a specification. Even if SpaceX internally builds something by the time you finish, that doesn't invalidate your work, for a few reasons:

- SpaceX's solution will be proprietary and single-operator. They will never open-source a bilateral negotiation protocol that works between SpaceX and, say, an Indian government satellite. Your research specifically targets the cross-operator, inter-agency problem — which no company has commercial incentive to solve for others.

- The research gap you're filling is a standards gap, not a product gap. Think of it like TCP/IP — the internet protocol that everyone uses. Individual companies (AOL, CompuServe) had their own proprietary networks before TCP/IP standardized everything. Your S2SCAN is research toward what a standard might look like — not a competing product.

- BTech research is evaluated on novelty of the research, not deployment. Even if ESA announces a semi-decentralized tool next year, your paper still contributes if your protocol handles something theirs doesn't (e.g., cross-operator incentive compatibility, or ZKP-based privacy).

- Your timeline is fine. A one-year simulation study producing a peer-reviewed protocol specification is a legitimate, citable contribution — regardless of what industry does in parallel.

Part 3 — The Real 100+ Variables for Negotiation

This is the best part of your question. You're right that the basic version (fuel %, maneuver cost, mission priority) is too thin. Here are the actual, real-world categories of variables that would determine who moves — organized by the real types of satellites you mentioned.

Category A — Physical Orbital Characteristics

These describe where the satellite is and how it moves:

| # | Variable | Why It Matters |
| --- | --- | --- |
| 1 | Current orbital altitude (km) | Determines atmospheric drag, maneuver energy needed |
| 2 | Orbital inclination (degrees) | In-plane vs. out-of-plane maneuvers cost very different amounts |
| 3 | Eccentricity | Elliptical orbits have different maneuver windows than circular |
| 4 | Right ascension of ascending node | Defines the orbital plane orientation |
| 5 | Argument of perigee | Where in the ellipse the satellite currently is |
| 6 | Current velocity (km/s) | Faster = more fuel to change trajectory |
| 7 | Time to closest approach (seconds) | How much time is actually available |
| 8 | Miss distance at TCA (meters) | Closer = more dangerous |
| 9 | Relative velocity at closest approach | Higher = more destructive collision |
| 10 | Conjunction geometry (radial/in-track/cross-track) | Determines which direction to maneuver |

Category B — Propulsion and Fuel

| # | Variable | Why It Matters |
| --- | --- | --- |
| 11 | Remaining propellant mass (kg) | Total fuel budget |
| 12 | Propellant category (LOW/MED/HIGH/CRITICAL) | Categorical disclosure for privacy |
| 13 | Specific impulse (Isp) of thruster | How efficiently fuel converts to thrust |
| 14 | Thruster type | Chemical, electric (Hall), cold gas — massively different Δv capability |
| 15 | Maximum thrust force (N) | How fast the maneuver can be executed |
| 16 | Maneuver Δv required (m/s) | The actual cost of this specific avoidance |
| 17 | Minimum Δv capability | Some thrusters can't do micro-burns |
| 18 | Thruster operational status | Is the thruster currently healthy? |
| 19 | Number of remaining maneuver budgets | Some satellites have a hard limit on lifetime maneuvers |
| 20 | Fuel-to-mission-life ratio | How many months of life would this maneuver consume? |

Category C — Mission Status and Criticality

| # | Variable | Why It Matters |
| --- | --- | --- |
| 21 | Mission type | Communications, Earth observation, navigation, science |
| 22 | Mission phase | Launch/early orbit, operational, end-of-life — different priorities |
| 23 | Remaining mission lifetime (months) | End-of-life satellite can absorb more cost |
| 24 | Data collection criticality | Is there an ongoing time-sensitive observation window? |
| 25 | Service commitment level | Is this satellite currently serving paying customers? |
| 26 | Redundancy in constellation | Is there a backup satellite in the same slot? |
| 27 | Ground contact window upcoming | If maneuvering now cuts the next downlink, that has a cost |
| 28 | Payload operational status | Is the instrument/camera/transponder currently active? |
| 29 | Time since last maneuver | Some systems need settling time after a burn |
| 30 | Scheduled maintenance window | Is there a planned maneuver already queued? |

Category D — Satellite Class (Your Specific Examples — Very Important)

This is where your real-world types come in. Each class has hard constraints that must override the cost calculation:

D1 — Geostationary (GEO) Satellites — "Always Meant to Be Still Relative to Earth"

| # | Variable | Constraint |
| --- | --- | --- |
| 31 | GEO slot assignment (degrees East/West) | A GEO satellite cannot freely move — its slot is licensed by ITU |
| 32 | Station-keeping box limits (±0.05°) | Any maneuver outside the box violates international treaty |
| 33 | ITU license constraint flag | If moving would violate ITU allocation → CANNOT MOVE, PERIOD |
| 34 | Adjacent slot separation | Moving risks interfering with neighbors |
| 35 | Apogee kick motor status | GEO satellites have a separate raising motor — not always available |

Implication for your protocol: A GEO satellite that receives a collision negotiation must immediately declare its station-keeping constraint. The protocol must classify it as mobility-limited, which is a hard flag that overrides any cost calculation. The other satellite must move, no negotiation.

D2 — Government / Military Satellites

| # | Variable | Constraint |
| --- | --- | --- |
| 36 | Operator sovereignty class | Government, civil, military, commercial |
| 37 | Classification level | Can it share trajectory data at all? |
| 38 | Mission sensitivity flag | Intelligence satellite might not confirm its exact purpose |
| 39 | Diplomatic status | Some satellites have legal immunity from commands by foreign entities |
| 40 | Authorization chain | Government satellites may require human approval before any maneuver |
| 41 | Maneuver authority level | Can the satellite maneuver autonomously, or does it need ground uplink? |
| 42 | Treaty obligations | Some satellites (e.g., Open Skies Treaty) have specific operational rules |

Implication for your protocol: A classified government satellite might respond with only: NEGOTIATION DECLINED — SOVEREIGNTY CONSTRAINT. Your protocol must handle this gracefully — the other satellite auto-maneuvers. This is a real case: if an Indian ISRO satellite and a classified US NRO reconnaissance satellite are on a collision course, the NRO satellite will never tell ISRO anything about its state. Your protocol must have a defined behavior for this.

D3 — Formation Flying / Constellation Clusters — "3 Connected Together"

This is your most interesting class. Formation flying means satellites intentionally maintain precise relative positions — like ESA's Cluster mission (4 satellites in a tetrahedral formation), or GRACE (2 satellites precisely 220 km apart measuring gravity).

| # | Variable | Constraint |
| --- | --- | --- |
| 43 | Formation role | Leader, follower, reference node |
| 44 | Formation geometry type | String-of-pearls, tetrahedron, triangle, in-track |
| 45 | Inter-satellite separation target (m) | The formation must maintain this within tolerance |
| 46 | Formation coherence constraint | If Satellite A moves, does it break the formation for B and C? |
| 47 | Formation maneuver coupling | Can A maneuver independently or must all 3 maneuver together? |
| 48 | Formation operator | Same operator for all? If yes, internal coordination is possible |
| 49 | Formation re-acquisition cost | After breaking formation to avoid collision, how much fuel to re-form? |
| 50 | Formation scientific objective | Some formations are conducting experiments requiring exact geometry |
| 51 | Time to re-acquire formation after maneuver | Could be hours or days — this is a major cost |

Implication for your protocol: If Satellite A is part of a 3-satellite formation, a maneuver by A doesn't just cost A's fuel — it costs the fuel of all three satellites to re-acquire. Your negotiation weight must account for this formation disruption cost, which could be 3× or 10× the naive calculation. This is a variable no basic model includes.

D4 — Non-Maneuverable Satellites

| # | Variable | Constraint |
| --- | --- | --- |
| 52 | Propulsion system present | Yes/No — this is a hard constraint |
| 53 | Attitude control only | Can change orientation but not orbit |
| 54 | Dead/defunct status | Cannot maneuver at all |
| 55 | Drag sail deployed | Passive deorbit in progress |

Implication: If one satellite cannot maneuver at all, the protocol must immediately assign 100% responsibility to the other party. No negotiation — the non-maneuverable satellite just needs to inform the other party of its status.

Category E — Communication and Trust Variables

| # | Variable | Why It Matters |
| --- | --- | --- |
| 56 | ISL link quality (SNR, dB) | Poor link → messages might be lost → more fault tolerance needed |
| 57 | Current communication mode | Direct S2S, relay through ground, relay through third satellite |
| 58 | Message latency (ms) | How fresh is the state information you received? |
| 59 | Number of prior negotiation interactions with this satellite | Reputation — have they lied before? |
| 60 | Authentication certificate validity | Is the cert expired or revoked? |
| 61 | Time since last verified contact | Freshness of identity verification |
| 62 | Protocol version compatibility | Does the other satellite support S2SCAN v1 or v2? |
| 63 | Message integrity verified | Have all received messages passed signature checks? |
| 64 | Commitment scheme verification status | Has the other satellite's fuel commitment been verifiable? |
| 65 | Communication blackout window upcoming | Will the link die in 30 seconds? Rush to agreement needed |

Category F — Collision Risk Assessment Variables

| # | Variable | Why It Matters |
| --- | --- | --- |
| 66 | Probability of collision (Pc) | The primary trigger |
| 67 | Uncertainty in Pc estimate | Low-confidence estimates require more conservative action |
| 68 | Position uncertainty (covariance) | How accurate is the orbital prediction? |
| 69 | Velocity uncertainty | Error in speed measurement propagates to TCA prediction |
| 70 | Combined object cross-section (m²) | Bigger objects = higher Pc at same miss distance |
| 71 | Debris cloud risk (secondary) | Would a collision create debris endangering other satellites? |
| 72 | Number of third-party satellites at risk | If these two collide, who else gets hit? |
| 73 | Time horizon of conjunction prediction | 7-day predictions are less reliable than 1-day |
| 74 | Source of conjunction data | USSPACECOM CDM vs. commercial tracking vs. onboard radar |
| 75 | Pc update trajectory | Is the risk rising, falling, or stable? |

Category G — Post-Maneuver and Strategic Variables

| # | Variable | Why It Matters |
| --- | --- | --- |
| 76 | Post-maneuver drift rate | After moving, how far will the satellite drift from nominal orbit? |
| 77 | Re-circularization cost | Getting back to nominal orbit after avoidance |
| 78 | Next planned orbital maneuver | Might the existing planned maneuver already handle the conjunction? |
| 79 | Slot re-acquisition time | For GEO: how long to return to licensed position |
| 80 | Maneuver window start/end | Orbital mechanics constrain when burns can happen |
| 81 | Ground exclusion zones | Some maneuvers are forbidden when satellite is over certain regions |
| 82 | Sun angle constraint | Solar panels need to face sun — limits maneuver direction |
| 83 | Thermal constraint | Thrusters have heat limits — can't fire too long |
| 84 | Eclipse status | Satellites in shadow have limited power |
| 85 | Battery charge level | Powers the thruster — low battery limits maneuver magnitude |

Category H — Operator and Commercial Context

| # | Variable | Why It Matters |
| --- | --- | --- |
| 86 | Operator nationality | Different regulations, different legal obligations |
| 87 | Regulatory framework | ITU, FCC, national space agency — different rules |
| 88 | Insurance status | Insured satellite may accept more risk; uninsured may demand avoidance |
| 89 | SLA commitments | Are customers expecting service right now? |
| 90 | Operator maneuver authorization policy | Does the satellite have autonomous maneuver authority or need ground approval? |
| 91 | Bilateral treaty between operators | Some operators have pre-agreed coordination agreements |
| 92 | Dispute resolution mechanism | If negotiation fails, who arbitrates? |
| 93 | Maneuver cost liability | Who bears responsibility if the maneuver causes a secondary issue? |
| 94 | Historical compliance record | Has this operator complied with past coordination requests? |
| 95 | Shared constellation membership | Do both satellites belong to a shared framework (e.g., both ITU signatories)? |

Category I — Extreme Edge Cases (Makes Your Research Interesting)

| # | Variable | Scenario |
| --- | --- | --- |
| 96 | Both satellites non-maneuverable | Protocol must immediately escalate to ground emergency |
| 97 | Both satellites critically low on fuel | Negotiation must find the least-bad option |
| 98 | Three-way conjunction | A, B, and C all close to each other — bilateral protocol extends to multi-party |
| 99 | Asymmetric threat perception | A thinks risk is 1-in-10,000; B's data says 1-in-1,000,000 — who's right? |
| 100 | One satellite in its last 48 hours of life | De-orbiting satellite should yield, but protocol must recognize this state |
| 101 | Communication restored after partial agreement | A committed, B didn't receive — what's the safe state? |
| 102 | Spoofed identity detected mid-negotiation | Protocol must abort and fallback |
| 103 | Formation satellite collision with non-formation satellite | Multi-satellite cost calculation applies |
| 104 | Both GEO satellites in adjacent slots | Neither can move significantly — requires ground escalation flag |
| 105 | Satellite carrying humans (crewed vehicle) | Human safety overrides all cost calculations — other satellite always moves |

How the Negotiation Weight Actually Works

With these variables, here's how your algorithm computes the negotiation weight (who should move):

NEGOTIATION_WEIGHT(satellite_i) =

α × (Fuel_Cost_i / Max_Fuel_Cost)          ← cheaper to move = lower weight

+ β × (1 - Fuel_Remaining_i / Max_Fuel)      ← more fuel = lower weight

+ γ × Mission_Priority_i                      ← higher priority = higher weight (protected)

+ δ × Formation_Disruption_Cost_i             ← formation satellite gets high weight

+ ε × Mobility_Constraint_i                   ← GEO, non-maneuverable = maximum weight (can't move)

+ ζ × Sovereignty_Flag_i                      ← government satellite = escalate to ground

+ η × Post_Maneuver_Recovery_Cost_i           ← if recovery is expensive, protect this satellite

The satellite with the LOWER weight moves. Constraints (GEO slot, no propulsion, government sovereignty) set weight to ∞ — meaning the other satellite must move regardless.

This is your actual research contribution — defining these weights, testing different α/β/γ values in simulation, and showing which weighting scheme produces the fairest outcomes across a realistic distribution of satellite types and conjunction scenarios.

Project Milestones — S2SCAN Protocol

Month 1 — Learn, Understand, and Define Everything Precisely

Goal by end of month: You can explain every component of your project to someone who has never heard of it. You have a written protocol specification document (not code — a spec, like a blueprint). You know exactly what you are building.

Week 1 — Orbital Mechanics (Just Enough, Not Too Deep)

You don't need a physics degree. You need to understand enough to build the simulation and not make embarrassing mistakes in the paper.

What to read/learn:

- TLE format — understand what each line means. A TLE is just two lines of numbers that describe an orbit. Learn: epoch, inclination, eccentricity, RAAN, argument of perigee, mean anomaly, mean motion. You don't need to derive them — just understand what each field represents and how to use Skyfield to load them.

- Skyfield library documentation — specifically EarthSatellite, .at(), .position_of_radec(), and computing distance between two satellite objects. Run the examples. Get comfortable.

- Conjunction basics — what is Time of Closest Approach (TCA), what is miss distance, what is Probability of Collision (Pc). Read NASA's CARA primer — it's publicly available and written in plain English. You need to understand how Pc is calculated conceptually (it's basically a Gaussian distribution over position uncertainty — two bell curves overlapping in space).

- Orbital regimes — LEO, MEO, GEO. Understand what GEO station-keeping means (why GEO satellites can't just move freely). Read one page on formation flying to understand why breaking a formation has a re-acquisition cost.

Deliverable at end of Week 1:
A 2-page written summary: "What is a conjunction, how is Pc calculated, what are the 4 satellite classes we are modelling, and why does each class have different mobility constraints." This forces you to actually understand it, not just skim it.

Week 2 — Distributed Systems and Protocol Design

What to read/learn:

- Two-Phase Commit (2PC) — this is the classic distributed systems protocol for getting two nodes to agree on something. Read the Wikipedia article and one tutorial. Your negotiation protocol is structurally similar to 2PC. Understanding 2PC means you already understand the core idea behind your protocol.

- Finite State Machines (FSMs) — read any CS textbook chapter on FSMs. Your protocol is literally an FSM. States: IDLE → DISCOVERY → AUTH → EXCHANGE → NEGOTIATE → COMMIT → EXECUTE → VERIFY → DONE (or ABORT at any point). Draw it on paper.

- Timeout and retry logic — read one article on how protocols handle message loss. The key insight: every state has a timer. If the timer expires before a response arrives, you transition to the fallback state.

- Byzantine Fault Tolerance basics — read just the introduction section of the original Lamport paper (it's famously written to be readable). You don't need to implement BFT — just understand what it means for a node to behave maliciously and how protocols defend against it.

- Practical: Python asyncio — your simulation will simulate concurrent satellites. Spend one day learning async/await in Python. This is how you'll simulate two satellites running their protocol simultaneously without actually using threads.

Deliverable at end of Week 2:
Draw the full FSM for your protocol on paper (or in draw.io). Every state, every transition, every timeout edge, every abort condition. This becomes Figure 1 in your paper.

Week 3 — Cryptography (The Pieces You Actually Need)

You do not need to learn all of cryptography. You need exactly four things:

What to read/learn:

- Public-key cryptography + digital signatures — read the "how RSA works" explainer on Khan Academy or Computerphile on YouTube. Understand: public key, private key, signing a message, verifying a signature. In Python, use the cryptography library (pip install cryptography). Practice: generate a key pair, sign a message, verify it. This takes one afternoon.

- Hash functions + commitment schemes — a commitment scheme is: commit = SHA256(value + nonce). The sender sends the commit. Later reveals value + nonce. Anyone can check that SHA256(value + nonce) == commit. This proves you committed to a value without revealing it. Practice: implement this in 10 lines of Python.

- Nonces and replay protection — read one short article on why timestamps + nonces prevent replay attacks (sending an old valid message again to confuse the receiver).

- Zero-Knowledge Proofs — conceptual only — read the "Ali Baba cave" explanation of ZKPs (it's a 5-minute read, available everywhere). You don't implement ZKPs in the simulation — you simulate the output of a ZKP (i.e., your code says "B proves fuel > threshold without revealing exact value" — the cryptographic machinery is abstracted). If you want to go further, look at Python's py_ecc library, but this is optional.

- DO NOT fall into the rabbit hole of — elliptic curve math derivations, fully homomorphic encryption, or implementing your own crypto primitives. Use existing libraries.

Deliverable at end of Week 3:
A working Python script (50–100 lines) that simulates the security handshake between two "satellite objects": generate keys, sign a conjunction alert message, verify it, then do a commitment scheme for fuel level. This is the seed of your security layer.

Week 4 — Game Theory and the Negotiation Algorithm

What to read/learn:

- Nash Equilibrium — conceptual — read the prisoner's dilemma and the explanation of Nash equilibrium. You need to understand: each player picks the best strategy assuming the other plays their best strategy. In your case: what's the "best" strategy for Satellite A given what B does?

- Mechanism design — read the one-page introduction to mechanism design on any economics explainer site. The key idea: design the rules of the game so that honest behavior is the best strategy. This is what makes your protocol "incentive-compatible."

- Your weighting formula — spend two days formalizing your negotiation weight formula (from the 105 variables). Decide: which variables go into the formula, what weights (α, β, γ...) do you use, and how do you normalize everything to the same scale? Write this as a 1-page math spec. Don't worry about whether the weights are perfect — you'll tune them in simulation.

- Read one paper: "Game-theoretic analysis of satellite collision avoidance" — search Google Scholar for this. Read the abstract, introduction, and conclusion. You don't need to understand every equation. You need to understand how they framed the problem and cite it correctly.

Deliverable at end of Week 4:
A complete written protocol specification document — 5 to 8 pages, no code. This covers: all 6 stages of S2SCAN, every message type and its fields, the FSM diagram, the weighting formula, the fault conditions and their fallbacks, and the variable categories. This document is what you build from. It also becomes the core of Section 3 of your paper.

What You Have in Hand After Month 1

- Full understanding of the orbital, distributed systems, crypto, and game theory pieces

- A drawn FSM that is the protocol

- A working Python security handshake (seed code)

- A complete written protocol spec document

- A clear division of who builds what in Months 2 and 3

Month 2 — Build the Simulator Core

Goal by end of month: A working simulation where two virtual satellites can detect a conjunction, run through the S2SCAN negotiation protocol, and reach a decision. No fancy UI needed — command-line output is fine.

Week 5 — Orbital Data Pipeline (Person B primary)

What to build:

- TLE loader — write a Python script that fetches real Starlink TLE data from celestrak.org (free, public API) and loads it into Skyfield EarthSatellite objects. You'll end up with ~200 satellite objects to work with (don't try all 8,000 Starlinks — pick a manageable subset).

- Position propagator — for each satellite, compute its (x, y, z) position at every minute over a 24-hour window. Store these as a numpy array. This becomes your simulation timeline.

- Conjunction detector — iterate over all satellite pairs, compute distance at each timestep. If distance drops below your threshold (say, 5 km), flag a conjunction event. Record: satellite A ID, satellite B ID, TCA timestamp, miss distance, relative velocity.

- Conjunction dataset — run this on your 200 satellites over 24 hours. You will find real, actual conjunctions in the Starlink TLE data. These become your test cases. Generate a CSV of conjunction events.

What to learn this week:
Skyfield's EarthSatellite, numpy broadcasting for fast distance computation, and basic pandas for storing the conjunction dataset.

Deliverable: A script that outputs a CSV of real conjunction events from real Starlink data. Something like: [SAT_A, SAT_B, TCA, miss_dist_km, relative_vel_km_s]. This is your ground truth dataset.

Week 6 — Satellite Agent Model (All three)

What to build:

A Python class SatelliteAgent that represents a satellite participating in S2SCAN. This is the most important class in your codebase.

class SatelliteAgent:

# Identity

- satellite_id

- operator_id

- public_key / private_key

# Orbital state (loaded from TLE)

- orbital_elements

- current_position()

- current_velocity()

# Satellite class

- sat_class: Enum(LEO_COMM, GEO, FORMATION, GOVERNMENT, NON_MANEUVERABLE)

- formation_group_id (if applicable)

- formation_role (LEADER / FOLLOWER / REFERENCE)

- mobility_constraint: bool  # GEO or non-maneuverable = True

- sovereignty_flag: bool     # government = True

# Fuel and propulsion

- fuel_remaining_pct       # internal truth (never shared directly)

- fuel_category()          # LOW/MED/HIGH/CRITICAL — what gets shared

- thruster_type

- max_delta_v()

- compute_maneuver_cost(delta_v)

# Mission

- mission_priority         # 1-10 scale

- mission_phase            # OPERATIONAL / END_OF_LIFE / etc.

- redundancy_available: bool

# Protocol state

- protocol_state: FSM state

- negotiate(other_agent)   # runs the protocol

# Negotiation

- compute_negotiation_weight()

- generate_commitment(value)

- verify_commitment(commit, value, nonce)

- sign_message(msg)

- verify_signature(msg, sig, pub_key)

Build this class. Write unit tests for every method. Make sure compute_negotiation_weight() correctly applies the formula with all the variable categories.

Deliverable: satellite_agent.py with full class, unit tests passing, and a test script that creates two agents with different classes (e.g., GEO vs. LEO) and prints their computed weights.

Week 7 — Protocol Engine (Person A primary)

What to build:

The actual S2SCAN protocol logic. This runs on top of two SatelliteAgent objects.

- Message types as Python dataclasses:

- ConjunctionAlert(sender_id, timestamp, pc, tca, miss_dist, signature)

- AuthRequest(sender_id, nonce, public_key, signature)

- AuthResponse(sender_id, nonce, public_key, signature)

- StateExchange(sender_id, fuel_category, maneuver_capable, mobility_constraint,

- sovereignty_flag, formation_flag, formation_disruption_cost_category,

- fuel_commitment_hash, signature)

- ManeuverProposal(sender_id, proposed_mover, proposed_delta_v_category,

- justification_hash, signature)

- CounterProposal(sender_id, proposed_mover, counter_reason, signature)

- Accept(sender_id, agreed_mover, agreed_delta_v_category, signature)

- Reject(sender_id, reason, signature)

- ExecutionCommit(sender_id, maneuver_plan_hash, timestamp, signature)

- ExecutionConfirm(sender_id, executed, new_pc_estimate, signature)

- Abort(sender_id, reason, timestamp, signature)

- Protocol runner — a function run_negotiation(agent_a, agent_b, conjunction_event) that steps both agents through the FSM, exchanging messages. Simulate message loss with a configurable probability parameter (e.g., message_loss_prob=0.1 means 10% of messages get dropped).

- Fault injection — add flags: agent_b.lies_about_fuel = True (B claims CRITICAL when it's actually HIGH). The protocol should detect this at the commitment verification stage and trigger ABORT.

Deliverable: protocol_engine.py — running a negotiation between two agents and printing a message trace like:

[T+0.0s] A → B: ConjunctionAlert (Pc=1.2e-4, TCA=94s)

[T+0.1s] A → B: AuthRequest

[T+0.2s] B → A: AuthResponse [VERIFIED ✓]

[T+0.3s] A → B: StateExchange (fuel=LOW, mobile=YES, formation=NO)

[T+0.3s] B → A: StateExchange (fuel=HIGH, mobile=YES, formation=YES, disruption=HIGH)

[T+0.4s] A → B: ManeuverProposal (mover=B, reason=fuel+formation)

[T+0.5s] B → A: CounterProposal (mover=A, reason=formation_constraint)

[T+0.6s] A → B: ManeuverProposal (mover=A, accepted)

[T+0.7s] AGREEMENT: A maneuvers. B holds.

[T+0.8s] A → B: ExecutionCommit

[T+0.9s] B → A: ExecutionConfirm [new Pc=1.1e-9 ✓]

Week 8 — Baseline Implementations (Person C primary)

What to build:

Two comparison baselines that you'll measure your protocol against:

- Baseline 1 — Random assignment: When a conjunction is detected, randomly pick one satellite to maneuver. No negotiation. Measure: how often does the satellite with less fuel get picked? How often does the GEO satellite get picked (which should never happen)? This shows why naïve random assignment is bad.

- Baseline 2 — Ground-mediated (simulated): Simulate the current real-world process. Add a 15-minute delay (representing human processing time + uplink latency), during which the conjunction risk continues to evolve. If TCA < 15 minutes, the ground process fails to respond in time. Measure: how often does this happen given your real conjunction dataset?

- Your protocol (S2SCAN): Average negotiation time from your message trace logs.

Deliverable: Three runnable scripts (baseline_random.py, baseline_ground.py, s2scan_protocol.py) that all take the same conjunction event CSV as input and produce comparable output.

What You Have in Hand After Month 2

- A real conjunction event dataset from actual Starlink TLE data

- A full SatelliteAgent class that models all 4 satellite types

- A working message-trace protocol engine that runs S2SCAN end to end

- Two baselines to compare against

- Your first bug list — things that broke that you need to fix in Month 3

Month 3 — Run Experiments, Measure Everything, Fix What Breaks

Goal by end of month: Results. Actual numbers. Tables and graphs that answer your research questions. This is what goes in Section 4 of your paper.

Week 9 — Experiment Design and Setup (All three)

Before running anything, write down exactly what questions you are answering. These become your paper's research questions:

RQ1 — Correctness: Does S2SCAN always assign maneuver responsibility to the lower-cost satellite? Does it correctly give GEO satellites, non-maneuverable satellites, and formation satellites their right weight?

RQ2 — Efficiency: How many messages does S2SCAN exchange per negotiation on average? How long does it take (simulated time)? How does this compare to the 15-minute ground-mediated baseline?

RQ3 — Fault tolerance: Under message loss rates of 0%, 5%, 10%, 20%, 30% — does the protocol still reach a safe decision? What percentage fall back to the safe default?

RQ4 — Security: Under adversarial conditions (satellite lies about fuel, satellite sends tampered messages, satellite drops out mid-negotiation) — does the protocol detect and handle this correctly?

RQ5 — Fairness: Over repeated conjunctions between the same pair, does one satellite systematically bear all the cost? Or does responsibility rotate fairly based on state?

Week 10 — Run Experiments at Scale (Person B primary)

- Pull all conjunction events from your dataset (you might find 50–200 across the 200 Starlinks over 24 hours)

- For each conjunction event, generate randomized satellite agent pairs with random fuel levels, random classes (weighted by real-world distribution — most will be LEO commercial, some GEO, some formation, few government)

- Run S2SCAN on each pair. Log: who was assigned to maneuver, message count, negotiation latency, whether agreement was reached, whether commitment verification passed

- Run the same events through both baselines

- Repeat each experiment 100 times with different random seeds (Monte Carlo) to get statistically stable averages

- Vary message loss probability from 0% to 30% in 5% steps and re-run

Deliverable: Raw results CSV files. One row per conjunction event per run.

Week 11 — Adversarial and Edge Case Testing (Person C primary)

- Run the lying-satellite scenario: B claims CRITICAL fuel when it's HIGH. Does the commitment verification catch it? Over 1,000 runs, what's the detection rate?

- Run the drop-out scenario: B sends AuthResponse and then disappears. Does A correctly time out and apply the fallback (maneuver independently)?

- Run the GEO scenario: A is GEO, B is LEO. Does the protocol always assign B to maneuver?

- Run the formation scenario: A is part of a 3-satellite formation. Does the formation disruption cost correctly make A less likely to be assigned, and does the negotiation weight calculation reflect the 3× cost multiplier?

- Run the both-low-fuel scenario: A is CRITICAL, B is CRITICAL. Does the protocol reach a decision or deadlock? What's the fallback?

- Run the non-maneuverable scenario: B cannot maneuver. Does the protocol immediately assign A?

Deliverable: An adversarial test results table with detection rates, fallback rates, and correctness percentages for each scenario.

Week 12 — Analysis, Graphs, and Paper Writing Start (All three)

Graphs to produce:

- Line chart: message loss rate (x-axis) vs. successful agreement rate (y-axis) — your fault tolerance curve

- Bar chart: S2SCAN vs. Baseline Random vs. Baseline Ground — on fuel fairness and correctness

- Table: adversarial scenario → detection rate → fallback outcome

- Table: average message count and negotiation latency per satellite class combination (GEO-LEO, LEO-LEO, Formation-LEO, etc.)

- Histogram: distribution of negotiation outcomes across your 200 conjunction events

Paper sections to start drafting this week:

- Abstract (draft — will be rewritten)

- Introduction (why the problem matters — use your stats from earlier)

- Related Work (use your comparison table from the Word doc)

Deliverable: All graphs generated. Introduction and Related Work sections drafted. Results section started.

What You Have in Hand After Month 3

- Full experimental results across 5 research questions

- Adversarial test results showing security properties

- All graphs and tables ready to drop into the paper

- Introduction, Related Work, and Results sections drafted

Month 4 — Write, Polish, and Submit

Goal by end of month: A complete, submitted research paper.

Week 13 — Complete Paper Draft (All three, split by section)

- Person A writes: Protocol Design section (FSM diagram, message spec, fault states) — this is the longest and most technical section

- Person B writes: Experimental Setup + Simulation Environment section (TLE data, how agents were configured, what baselines were used)

- Person C writes: Results section (all the graphs and tables with written analysis of what they show)

- All three together revise: Introduction, Related Work, and Conclusion

Target paper length: 8–10 pages in IEEE two-column format (standard for CS conferences).

Week 14 — Internal Review and Fix Round

- Each person reads the other two sections they didn't write

- Fix inconsistencies (your protocol description and your results must describe the same thing)

- Add citations for every claim that needs one (use Google Scholar, cite the papers from your comparison table)

- Run the paper through Grammarly or similar for language

- Check: does every graph have axis labels, a title, and is referenced in the text?

- Check: is the research question answered by the results? If RQ1 asks about correctness and your results section never explicitly answers it — fix this

Week 15 — Target Venue Research and Submission Prep

Find your target venue. Options appropriate for a BTech research project:

- IEEE ICISC (International Conference on Intelligent Systems and Control) — accepts simulation-based CS work

- ICECCS (International Conference on Engineering of Complex Computer Systems) — distributed systems focus

- Arxiv preprint — submit for public visibility even before peer review; this gives you a citable URL immediately

- IJEAT or IJARCCE — Indian-based journals that accept solid simulation research

For any conference: read the submission guidelines, format your paper in their template (IEEE usually provides a LaTeX or Word template), and check page limits.

Week 16 — Final Polish and Submission

- Final proofread

- Ensure all figures are high-resolution

- Write the author bios (even for BTech papers, some venues require these)

- Submit

- Also submit to Arxiv the same day regardless of the conference outcome — this timestamps your work publicly

What You Have in Hand at the End of the Project

By the end of Month 4, you will have:

| Deliverable | Description |
| --- | --- |
| Protocol Specification Document | 5–8 page written spec of S2SCAN — every message, every state, every variable |
| Conjunction Event Dataset | Real Starlink conjunction events extracted from TLE data — reusable for future research |
| SatelliteAgent class | Python class modeling all 4 satellite types with 105+ variables — reusable simulator component |
| S2SCAN Protocol Engine | Full Python implementation of the negotiation protocol with message traces |
| Two Baseline Implementations | Random assignment and ground-mediated simulations for comparison |
| Experimental Results | Full results across 5 research questions, 1,000+ simulation runs, adversarial tests |
| Research Paper | Submitted to a peer-reviewed venue or live on Arxiv |
| GitHub Repository | Clean, documented codebase — something you can show in internship/job interviews |

The GitHub repo alone is valuable beyond the paper. A working satellite negotiation simulator built on real orbital data, in Python, is something you can explain in any CS interview — it demonstrates distributed systems, cryptography, algorithm design, and simulation engineering in one project.

DOUBT 2 –

The 4 Special Satellite Classes and How Your Protocol Handles Each

Class 1 — GEO Satellites ("Always Still, Moving With Earth")

Yes, these are called Geostationary satellites. They orbit at exactly 35,786 km altitude, moving at the same speed as Earth's rotation — so from the ground, they look completely stationary. Weather satellites, TV broadcast satellites, most military communications satellites are GEO.

The hard constraint: A GEO satellite has an internationally licensed orbital slot — a specific degree of longitude (like 74°E for India's GSAT satellites). Moving out of that slot even slightly violates ITU (International Telecommunication Union) treaty law and risks interfering with neighboring GEO satellites.

How your protocol handles it:

When a GEO satellite receives a conjunction alert, it immediately sends back a single flag in its StateExchange message:

mobility_constraint = TRUE

constraint_reason = GEO_SLOT_LOCKED

This is a hard override in the negotiation weight formula. The GEO satellite's weight is set to the maximum possible value (∞ in theory, a very large number in code). The negotiation algorithm immediately assigns 100% responsibility to the other satellite — no further negotiation on this point. The GEO satellite doesn't move. Period. The other satellite moves.

But what if BOTH satellites are GEO? Adjacent GEO slots sometimes have conjunction risks. In this case, both satellites declare mobility_constraint = TRUE. Your protocol detects this deadlock condition and sends a GROUND_ESCALATION_REQUIRED abort message — human operators must intervene. This is a known edge case your paper can discuss as a limitation.

Class 2 — Government / Military Satellites

These can physically maneuver, but they operate under sovereignty constraints — they may not be able to share trajectory data with a foreign satellite, may require human authorization before any autonomous maneuver, or may simply refuse to negotiate with civilian satellites.

The realistic behavior:

A classified government satellite will likely never even respond to your protocol properly. In practice, your protocol needs to handle three sub-cases:

| Situation | What Happens |
| --- | --- |
| Government satellite can negotiate (civil, like ISRO's EOS) | Participates normally, but may share less data (fuel category only, no exact numbers) |
| Government satellite cannot share data (classified NRO, military) | Responds with SOVEREIGNTY_CONSTRAINT — declines negotiation entirely |
| Government satellite cannot maneuver autonomously | Responds with GROUND_AUTHORIZATION_REQUIRED — needs human uplink before moving |

How your protocol handles it:

If the other satellite sends SOVEREIGNTY_CONSTRAINT, your satellite doesn't wait for a negotiation. It treats it exactly like a non-maneuverable satellite — the other (civilian/commercial) satellite assumes full responsibility and maneuvers unilaterally. The government satellite just needs to acknowledge the conjunction exists (even a one-bit "yes I see it too" is enough for safety — the commercial satellite then knows to move).

This is actually important for your paper — it means your protocol degrades gracefully. It doesn't require full participation to produce a safe outcome. Even a partially cooperative government satellite produces a better result than the current system (email).

Class 3 — Formation Flying ("3 Connected Together")

You described this exactly right. These are satellites that fly in precise geometric patterns — intentionally. Real examples:

- ESA Cluster mission — 4 satellites in a tetrahedral (3D triangle) formation, studying Earth's magnetosphere. They must maintain their exact relative positions for the science to work.

- GRACE / GRACE-FO — exactly 2 satellites, precisely 220 km apart, measuring Earth's gravity field by the distance between them. Moving one by even a few meters ruins the measurement.

- Future StarShield / military formations — 3 satellites in a triangle for triangulated targeting or communications relay.

The constraint: If Satellite A is in a formation of 3, and A maneuvers to avoid a collision, then:

- A drifts out of the formation geometry

- All 3 satellites now need to spend fuel to re-acquire the formation

- The science/mission is interrupted for potentially hours or days

- The real cost of A maneuvering is not just A's fuel — it's 3× fuel plus mission downtime

How your protocol handles it:

Satellite A sends in StateExchange:

formation_flag = TRUE

formation_size = 3

formation_role = FOLLOWER

formation_disruption_cost_category = HIGH

formation_reacquisition_time_hours = 6

Your negotiation weight formula then applies a formation multiplier:

effective_maneuver_cost(A) = A's own fuel cost × formation_size × reacquisition_penalty

So if A's raw fuel cost is LOW, but it's in a 3-satellite formation with HIGH reacquisition cost, the effective cost becomes HIGH or CRITICAL. This makes A much less likely to be assigned as the mover — the other satellite (which has no formation constraints) should move instead.

What if both satellites are in formations? Then both declare formation disruption costs, and the algorithm compares them. The formation with the cheaper re-acquisition (smaller formation, more fuel, less scientific sensitivity) gets assigned.

Class 4 — Non-Maneuverable Satellites

Simple but important. Some satellites have no propulsion at all — small CubeSats, old legacy satellites, debris-generating dead satellites. They physically cannot move.

How your protocol handles it:

StateExchange declares maneuver_capable = FALSE. This is the simplest case. The other satellite immediately gets 100% responsibility. No negotiation needed on this point — the protocol skips straight to COMMIT where the maneuverable satellite commits to moving. The non-maneuverable satellite just sends ExecutionConfirm acknowledging it saw the agreement.

How All 4 Classes Interact in One Formula

Here's how your negotiation weight formula treats each class — in plain terms:

Weight = base fuel/mission cost

+ if GEO          → set to MAX (can never move)

+ if non-maneuver → set to MAX (physically can't move)

+ if government   → check sovereignty flag → if blocked, set to MAX

+ if formation    → multiply base cost by (formation_size × reacquisition_penalty)

The satellite with the LOWER weight moves. If one satellite has MAX weight, the other moves, no negotiation on that point. If both have MAX weight (two GEOs, two non-maneuverable) → escalate to ground.

This is clean, implementable, and defensible in a paper — because each class constraint maps directly to a real-world physical or legal reason.