This is not a “satellites cannot communicate” project. Satellites already communicate, and autonomous collision avoidance already exists in some constellations.

The CSE problem is the missing software/protocol/decision layer between two independently controlled spacecraft: how they authenticate each other, exchange just enough information, negotiate, compute a fair decision, agree on a maneuver, and recover safely if communication fails.

Autonomous Satellite-to-Satellite Negotiation for Collision Avoidance

1. Project Overview

Low Earth Orbit (LEO) is rapidly becoming crowded with communication, Earth-observation, navigation, scientific, and military satellites. Large constellations such as Starlink are adding thousands of spacecraft, while other large constellations are also being deployed.

This creates a problem that is different from the traditional satellite-collision problem.

The traditional question is:

“Can we detect that two objects might collide?”

Modern Space Situational Awareness (SSA) systems are already quite good at doing this.

The emerging question is:

“Once two maneuverable satellites are identified as being on a collision course, how can they autonomously coordinate what to do?”

Today, collision avoidance is still largely based around operators, ground systems, conjunction messages, and separately planned maneuvers. Recent research shows that the field is moving toward onboard and semi-decentralized planning, but the complete problem of secure, autonomous, cross-operator negotiation between spacecraft remains open. A 2026 Stanford-led study, for example, specifically investigates semi-decentralized multi-spacecraft collision avoidance under intermittent communication.

Our project investigates the next layer:

Can two independently operated satellites communicate directly and autonomously negotiate a safe collision-avoidance action, while minimizing unnecessary fuel use and protecting sensitive information?

2. The Problem in Simple Terms

Imagine two satellites:

Satellite A

- Operator: Company A

- Has 20 kg of usable propellant

- Can maneuver relatively easily

Satellite B

- Operator: Company B

- Has 5 kg of usable propellant

- Has a more expensive maneuver

Their predicted trajectories show that they may come dangerously close.

Both satellites have a problem.

If A moves, A consumes fuel.

If B moves, B consumes fuel.

If both move independently, they could make unnecessary maneuvers or, worse, create another conflict.

So the real question becomes:

Who should move, by how much, and how should the two satellites agree on that decision?

And there is another problem.

Satellite A may not want to reveal:

- its exact orbit,

- its remaining fuel,

- its maneuver capability,

- its mission priorities,

- or other sensitive information.

Therefore, the system needs to solve several problems simultaneously:

- Detection

- Identification

- Authentication

- Secure communication

- Trajectory/risk information exchange

- Decision-making

- Fairness

- Maneuver planning

- Agreement

- Failure handling

That combination is where the CSE component becomes substantial.

3. Important Correction to Our Original Assumption

Our initial thought was:

“Ground stations can already tell where satellites are. Why can't satellites just talk to each other?”

This is a good starting question, but technically it is incomplete.

Satellites can already communicate with each other.

Inter-satellite links (ISLs) are an established technology. Space communication standards such as CCSDS Proximity-1 define protocols for spacecraft-to-spacecraft links, including data-link functions. CCSDS is even updating/reviewing Proximity-1 specifications in 2026.

Therefore:

Our project is NOT:

“Build communication between satellites.”

It is:

“Build an autonomous software protocol that uses satellite-to-satellite communication to negotiate collision avoidance between independently controlled spacecraft.”

That distinction is extremely important when presenting the project.

4. Why Does the Problem Exist If Ground Stations Can Track Satellites?

Because knowing where something is does not automatically tell you what everyone should do.

Consider this simplified chain:

Ground tracking

↓

Satellite A location

Satellite B location

↓

Conjunction analysis

↓

Potential collision

↓

Operator A analyzes situation

↓

Operator B analyzes situation

↓

They communicate

↓

They decide who moves

↓

Maneuver is calculated

↓

Command is sent

↓

Satellite executes maneuver

↓

New trajectory is checked

The tracking portion is relatively well established.

The difficult part is the coordination and decision layer.

And the number of conjunction events is growing. A 2026 study reports that the U.S. Space Force tracked more than 600,000 Conjunction Data Messages per day in 2023, while recent reporting based on SpaceX's FCC disclosures puts Starlink's collision-avoidance maneuvers at more than 355,000 between June 2025 and May 2026.

The important point is not that every alert causes a collision.

It is that the number of decisions that must be processed is increasing dramatically.

5. What Is a Conjunction?

A conjunction is a predicted close approach between two space objects.

It does not necessarily mean:

“These satellites will collide.”

Instead, it means:

“Based on current orbital estimates and uncertainty, these objects may come sufficiently close that collision risk needs to be evaluated.”

The system therefore has to consider:

- position,

- velocity,

- time,

- uncertainty,

- covariance,

- relative motion,

- predicted closest approach,

- collision probability.

A simplified representation is:

Pc=fxAxBΣAΣB

where:

- xA= state of satellite A

- xB= state of satellite B

- ΣA= uncertainty of A

- ΣB= uncertainty of B

The satellites therefore aren't negotiating based on perfect information.

They are negotiating under uncertainty.

That is an important research problem by itself.

6. What Is Currently Being Done?

Current collision avoidance can broadly be divided into several layers.

Layer 1 — Space Surveillance

Ground-based radar, telescopes, tracking networks, and other SSA systems observe space objects.

They estimate:

- orbit,

- position,

- velocity,

- uncertainty.

Layer 2 — Conjunction Assessment

The system predicts whether two objects will come close.

Operators receive information such as Conjunction Data Messages (CDMs).

Layer 3 — Risk Assessment

The operator determines:

Is the probability of collision high enough to justify a maneuver?

Different operators may have different risk thresholds and operational policies.

This is itself a problem. Research on “norms of behavior” has specifically highlighted the question of how collision avoidance should work when both satellites are maneuverable.

Layer 4 — Maneuver Planning

If a maneuver is required, the operator calculates an avoidance maneuver.

The objective is generally something like:

minΔv

subject to:

Pc<Psafe

while also maintaining mission constraints.

Layer 5 — Command

The chosen maneuver is sent to the spacecraft.

7. The Existing System's Main Weakness

The major weakness isn't simply communication latency.

It is coordination at scale.

Suppose:

Ground

|

-----------------

|       |       |

A       B       C

\       |      /

\      |     /

\     |    /

Multiple

conjunctions

Each operator can independently calculate its own maneuver.

But eventually we encounter:

What happens when multiple independently controlled satellites need to make decisions about the same physical space?

This becomes a distributed-systems problem.

And that is where our CSE background becomes relevant.

8. What Research Already Exists?

This is where we need to be careful.

Our project cannot claim:

“Nobody has researched autonomous satellite collision avoidance.”

That is false.

There is substantial research in each individual component.

The research gap appears when we combine the components.

9. Research Area 1 — Autonomous Collision Avoidance

Researchers are already developing algorithms that allow spacecraft to calculate avoidance maneuvers onboard.

For example, a 2025 paper proposed an Equivalent Linear Velocity Obstacle (ELVO) approach for autonomous avoidance of multiple debris objects, specifically targeting real-time decision-making and reduced online computational load.

This is important because it proves:

A satellite can potentially calculate an avoidance action without constantly asking Earth what to do.

What it does NOT solve

It primarily answers:

“How should this satellite avoid an object?”

Our question is:

“How should two autonomous satellites decide between themselves who should maneuver?”

Different problem.

10. Research Area 2 — Decentralized Multi-Spacecraft Planning

This is even closer to our project.

A 2026 Stanford study, “Semi-Decentralized Multi-Spacecraft Collision Avoidance under Communication Constraints,” models collision avoidance as a semi-decentralized partially observable decision problem.

It explicitly studies the difficulty of planning when spacecraft have intermittent information rather than continuous communication. Its reported simulations achieved near-centralized maneuver quality while using fewer synchronization events.

This is extremely relevant to us.

But notice the distinction.

The paper focuses on:

How can multiple spacecraft coordinate planning when communication is limited?

Our proposed system asks an additional question:

How can independently owned spacecraft establish a secure protocol for negotiating the actual maneuver decision?

So this paper is not something we can ignore.

It becomes one of our closest baseline papers.

11. Research Area 3 — Secure Collision-Probability Calculation

This is another major piece.

Satellite operators may not want to expose their exact orbital information.

A 2016 study by Hemenway et al. demonstrated secure multiparty computation for calculating satellite collision probabilities without directly sharing private trajectory information.

In simplified terms:

Satellite A

private orbit

|

| encrypted computation

|

+----------+

|

Collision risk

|

+----------+

|

|

Satellite B

private orbit

Neither side necessarily needs to reveal its complete private data.

2025: Encrypted collision probability

More recent work has pushed this further.

The 2025 Encrypted Pcresearch combines homomorphic encryption and multiparty computation to calculate collision probability while protecting sensitive orbital information.

This is extremely important for our project because it means:

Privacy-preserving orbital computation is already being researched.

Therefore we should NOT claim:

“We invented secure trajectory sharing.”

We haven't.

Instead:

We want to investigate how privacy-preserving orbital information exchange can become part of a real-time autonomous negotiation protocol.

That's a much stronger claim.

12. Research Area 4 — Game Theory

Game theory is also not new in spacecraft control.

Researchers have used game-theoretic models for satellite pursuit and collision avoidance.

For example, orbital pursuit-evasion research models satellites as players whose objectives conflict and calculates control strategies using differential games.

But this is not exactly our problem.

A pursuit-evasion game asks something like:

“How does one spacecraft evade another?”

Our problem is:

“Two spacecraft both want to avoid collision, but neither wants to unnecessarily spend its limited resources.”

That's closer to a cooperative bargaining problem.

13. Research Area 5 — Rules of the Road

There is also research and industry work around establishing rules for deciding who should maneuver.

Kayhan Space, for example, has operational documentation around maneuverability categories and “Rules of the Road,” including situations where bilateral discussions or pre-coordinated agreements are required.

Their operational system also supports coordinated avoidance maneuvers between operators.

This is important because it challenges another potential claim:

“Nobody is trying to automate coordination between operators.”

They are.

Therefore our research needs to move one level deeper:

Can this coordination become a standardized, secure, machine-to-machine protocol rather than primarily an operator/platform coordination process?

14. Comparison of Existing Research

| Research | Main Problem | Autonomous? | Decentralized? | Secure/Private Data? | Game Theory? | Direct Satellite Negotiation? | Our Difference |
| --- | --- | --- | --- | --- | --- | --- | --- |
| Hemenway et al. — Secure Pc | Private collision probability | No | Two-party computation | Yes | No | No | Add negotiation + decision |
| Suh et al. — Encrypted Pc | Privacy-preserving conjunction analysis | Limited | MPC architecture | Yes | No | No | Make it part of real-time negotiation |
| ELVO autonomous avoidance | Autonomous maneuver generation | Yes | Limited | No | No | No | Add second autonomous player |
| Stanford SDec-POMDP / RS-SDA* | Multi-spacecraft planning | Yes | Yes | No | Decision theory | Not as negotiation | Add security + operator incentives |
| Orbital game-theory research | Strategic spacecraft control | Yes | Potentially | No | Yes | No | Apply bargaining to fuel allocation |
| Kayhan coordination | Operator coordination | Automated platform | Partially | Operational security | Rules-based | Indirectly | Move coordination closer to spacecraft |
| CCSDS Proximity-1 | Spacecraft communication | N/A | Supports peer links | Protocol-dependent | No | Communication only | Build negotiation protocol above it |
| Our proposed system | Secure autonomous collision negotiation | Yes | Yes | Yes | Yes | Yes | Integrates all layers |

The important observation is:

The individual pieces exist. The integrated architecture is where the research opportunity lies.

15. So Where Exactly Is the Research Gap?

We can visualize existing research like this:

SATELLITE COLLISION AVOIDANCE

|

+-------------------+--------------------+

|                   |                    |

↓                   ↓                    ↓

Detection            Planning             Communication

|                   |                    |

|                   |                    |

↓                   ↓                    ↓

Mature             Active research      Mature/active

|

|

+------------------+

|

↓

Secure information

sharing

|

↓

Active research

|

↓

Game theory

|

↓

Active research

But what is missing is the complete chain:

SATELLITE A                         SATELLITE B

|                                  |

|                                  |

Detect threat                       Detect threat

|                                  |

+------------ SECURE --------------+

|

Authenticate

|

Exchange data

|

Estimate risk

|

Generate options

|

+------------+------------+

|                         |

A evaluates                B evaluates

|                         |

+------------+------------+

|

NEGOTIATE

|

Decide who moves

|

Agree on plan

|

Confirm receipt

|

Execute maneuver

|

Verify result

That complete protocol is our target.

16. The CSE Part — This Is Where Our Project Actually Lives

This is the part you were asking about.

If we merely build an orbital simulation:

“Satellite A moves away from satellite B.”

That is primarily an aerospace/control project.

It becomes a CSE project when we build the computational infrastructure that allows autonomous agents to coordinate.

Our system can contain at least six major CSE components.

16.1 Distributed Systems

There is no central server making the final decision.

Instead:

Satellite A Agent

↕

Satellite B Agent

Each satellite has:

- local state,

- local constraints,

- local decision logic,

- local computational resources.

This is fundamentally a distributed computing problem.

We have to deal with:

- message delays,

- dropped messages,

- inconsistent states,

- stale information,

- duplicate messages,

- conflicting decisions,

- synchronization,

- failure recovery.

These are classic distributed-systems problems.

17. 16.2 Computer Networks

The satellites need a communication protocol.

For example:

HELLO

↓

AUTHENTICATE

↓

CONJUNCTION_ALERT

↓

STATE_SUMMARY

↓

RISK_REQUEST

↓

RISK_RESPONSE

↓

MANEUVER_PROPOSAL

↓

COUNTER_PROPOSAL

↓

AGREEMENT

↓

ACKNOWLEDGEMENT

↓

EXECUTION

This becomes a custom application-layer protocol running over an inter-satellite link.

Existing standards such as Proximity-1 provide lower-level spacecraft communication capabilities, but they do not define our collision-negotiation logic.

That distinction is critical.

18. 16.3 Cybersecurity

This is probably one of the strongest CSE components.

Imagine a malicious satellite sends:

“I have only 1% fuel. You must move.”

If Satellite A believes it, B could manipulate A into spending fuel.

Or a malicious entity could send:

“Collision probability = 0.”

when the actual risk is high.

Therefore we need:

Authentication

Who sent the message?

Integrity

Was the message modified?

Confidentiality

Can someone else read it?

Replay protection

Can an attacker resend an old maneuver agreement?

Non-repudiation

Can we prove what was agreed?

Key management

How do satellites establish and rotate cryptographic keys?

This turns the project into a real cybersecurity problem.

19. 16.4 Cryptography

We could investigate a lightweight cryptographic protocol.

For example:

A → B:

Satellite_ID_A

Timestamp

Nonce

Certificate

Signature

B → A:

Satellite_ID_B

Timestamp

Nonce

Certificate

Signature

Then establish a session key.

After that:

Encrypted_Message =

AES-GCM(

session_key,

negotiation_data

)

The exact cryptographic design needs research.

We shouldn't automatically use extremely expensive homomorphic encryption for everything.

A key CSE question is:

What is the minimum cryptographic protection required while still being computationally feasible on spacecraft-class hardware?

20. 16.5 Multi-Agent Systems

Each satellite becomes an autonomous agent.

Satellite A Agent

/        |        \

State     Risk     Utility

|          |         |

+----------+---------+

|

Decision

|

Negotiation

↕

Negotiation

|

Satellite B Agent

Each agent observes its own state and some information from the other satellite.

This is a multi-agent system.

21. 16.6 Game Theory

Now we reach one of the most interesting parts.

Suppose:

Satellite A

Fuel cost:

CA=4

Satellite B

Fuel cost:

CB=1

If B moves, the total cost is:

C=1

If A moves:

C=4

So purely minimizing fuel suggests:

B should move.

But suppose B has only 2% fuel remaining while A has 70%.

Then the situation changes.

So we need a utility function.

For example:

Ui=wsSi-wfFi-wmMi-wrRi

where:

- Si= safety benefit

- Fi= fuel cost

- Mi= mission disruption

- Ri= residual collision risk

The two agents then attempt to reach an agreement.

This is where we can investigate:

- Nash bargaining

- weighted utility

- cost-sharing

- auctions

- cooperative game theory

- Pareto optimality

- mechanism design

22. A Simple Negotiation Example

Suppose:

Satellite A

Fuel: 80%

Maneuver cost: 5 units

Satellite B

Fuel: 20%

Maneuver cost: 2 units

A naive system might say:

B should move because B needs only 2 units.

But B might have much less remaining fuel.

A more intelligent system could calculate:

A's relative fuel sacrifice = 5 / available_fuel

B's relative fuel sacrifice = 2 / available_fuel

Suppose:

A = 5 / 80 = 0.0625

B = 2 / 20 = 0.10

Now A's maneuver is actually less damaging relative to its resources.

Therefore:

A should maneuver.

This is the sort of decision logic we can research.

23. The Most Interesting CSE Problem: What Should They Reveal?

This is where the project gets much more sophisticated.

Suppose A sends:

Fuel remaining = 2 kg

B now knows A is weak.

B could exploit that.

Therefore maybe A shouldn't reveal exact fuel.

Instead A could send:

Fuel class = LOW

Maneuver cost class = HIGH

or:

Available maneuver capability:

LOW / MEDIUM / HIGH

Now we have an interesting research question:

Can two satellites reach a good decision without revealing unnecessary private information?

This connects:

- distributed systems,

- cryptography,

- privacy,

- game theory,

- optimization.

That is a strong CSE research direction.

24. Proposed Architecture

Our first conceptual architecture could look like this:

┌─────────────────────────┐

│      SPACECRAFT A       │

│                         │

│  Orbit State            │

│  Fuel State             │

│  Mission Constraints    │

│  Maneuver Capability    │

│          │              │

│          ▼              │

│  Collision Agent        │

│          │              │

│  Negotiation Engine     │

│          │              │

│  Crypto Engine          │

└──────────┬──────────────┘

│

Secure ISL

│

│

┌──────────▼──────────────┐

│      SPACECRAFT B       │

│                         │

│  Crypto Engine          │

│          │              │

│  Negotiation Engine     │

│          │              │

│  Collision Agent        │

│          │              │

│  Orbit State            │

│  Fuel State             │

│  Mission Constraints    │

│                         │

└─────────────────────────┘

25. Proposed Software Architecture

We can make the CSE architecture more concrete.

┌──────────────────────────────────────────┐

│              SATELLITE AGENT             │

├──────────────────────────────────────────┤

│                                          │

│  1. State Manager                        │

│     ├── Position                          │

│     ├── Velocity                          │

│     ├── Uncertainty                       │

│     └── Fuel                              │

│                                          │

│  2. Threat Detector                       │

│     └── Conjunction Detection             │

│                                          │

│  3. Risk Engine                           │

│     └── Collision Probability             │

│                                          │

│  4. Maneuver Planner                      │

│     └── Candidate Maneuvers                │

│                                          │

│  5. Negotiation Engine                     │

│     ├── Proposal                          │

│     ├── Counterproposal                   │

│     ├── Utility Evaluation                │

│     └── Agreement                         │

│                                          │

│  6. Security Layer                        │

│     ├── Authentication                    │

│     ├── Encryption                        │

│     ├── Signatures                        │

│     └── Replay Protection                 │

│                                          │

│  7. Communication Layer                   │

│     └── Inter-Satellite Protocol           │

│                                          │

│  8. Safety Controller                     │

│     ├── Validate Agreement                │

│     ├── Reject Unsafe Plan                │

│     └── Emergency Fallback                │

│                                          │

└──────────────────────────────────────────┘

This is unmistakably a computer science/software systems project, with orbital mechanics acting as the domain.

26. Proposed Negotiation Protocol

We could call the protocol something temporary like:

ASNP — Autonomous Satellite Negotiation Protocol

The name can change later.

A simplified interaction could be:

A                                      B

│                                      │

│──── HELLO + ID ────────────────────>│

│                                      │

│<─── AUTHENTICATION ─────────────────│

│                                      │

│──── CONJUNCTION ALERT ─────────────>│

│                                      │

│<─── ACKNOWLEDGEMENT ────────────────│

│                                      │

│──── RISK SUMMARY ──────────────────>│

│                                      │

│<─── RISK SUMMARY ───────────────────│

│                                      │

│──── MANEUVER PROPOSAL ─────────────>│

│                                      │

│<─── COUNTERPROPOSAL ────────────────│

│                                      │

│──── ACCEPT / NEW PROPOSAL ─────────>│

│                                      │

│<─── AGREEMENT ──────────────────────│

│                                      │

│──── SIGNED AGREEMENT ──────────────>│

│                                      │

│<─── EXECUTION ACK ──────────────────│

│                                      │

The agreement might contain:

Agreement ID

Satellite A ID

Satellite B ID

Chosen maneuver:

A / B / both

Maneuver vector:

ΔVx

ΔVy

ΔVz

Execution time

Expected miss distance

Risk threshold

Validity period

Timestamp

Nonce

Digital signatures

27. Why Do We Need an Agreement?

This sounds unnecessary until we consider this situation:

A decides: "I will move."

B decides independently: "I will move too."

Now both maneuver.

That could result in:

- unnecessary fuel expenditure,

- unexpected trajectory changes,

- secondary conjunctions,

- coordination failure.

The protocol therefore needs something like:

Commit → Confirm → Execute

rather than:

Decide → Execute

28. Two-Phase Negotiation

A possible design:

Phase 1 — Proposal

Satellite A proposes:

A will maneuver

Δv = 0.8 m/s

T = 14:32:10

B evaluates it.

Phase 2 — Commitment

B sends:

ACCEPT

Agreement_ID = 82491

A confirms.

Only after both sides have acknowledged the agreement:

EXECUTE

This resembles distributed transaction concepts.

That is another strong CSE connection.

29. Failure Scenario

What if:

A → B: ACCEPT

but the message never arrives?

A might think:

“B agreed.”

B might think:

“I never agreed.”

So the protocol needs:

- timeout,

- retransmission,

- sequence numbers,

- unique transaction IDs,

- signed messages,

- rollback/fallback behavior.

This is basically distributed transaction handling under unreliable communication.

30. Emergency Mode

We cannot assume communication will always work.

Suppose:

T_collision = 3 minutes

and the satellites lose their communication link.

They cannot wait forever.

So the system needs a fallback hierarchy:

Level 1

Direct negotiation

↓ failure

Level 2

Predefined Rules of the Road

↓ failure

Level 3

Local autonomous maneuver

↓ failure

Level 4

Ground/operator intervention

This hybrid architecture is more realistic than claiming that Earth will become completely unnecessary.

31. What Exactly Are We Proposing to Build?

We should not initially claim that we are going to put this system on an actual satellite.

Our first target should be a software prototype.

Proposed prototype:

A simulation containing:

Satellite A Agent

↕

Secure Network

↕

Satellite B Agent

Each agent has:

- simulated orbit,

- simulated fuel,

- maneuver capabilities,

- uncertainty,

- cryptographic identity,

- utility function,

- negotiation algorithm.

32. Simulation Environment

The simulator can generate scenarios such as:

Scenario 1

A and B approach each other.

A has more fuel.

B has less fuel.

Who moves?

Scenario 2

Both have equal fuel.

Which satellite moves?

Scenario 3

A has a cheaper maneuver but high mission disruption.

B has a slightly more expensive maneuver but no mission disruption.

Who moves?

Scenario 4

A lies about its fuel.

Can B detect manipulation?

Scenario 5

Communication drops.

What happens?

Scenario 6

A sends an old maneuver agreement.

Can B detect replay?

Scenario 7

Three satellites converge.

A ↘

X

B ↗

C →

Can the system prevent conflicting maneuvers?

This takes the project toward swarm coordination rather than merely two-agent negotiation.

33. What We Can Measure

The project needs measurable results.

We can compare our system against conventional approaches.

Metric 1 — Collision risk

Pc

Lower is better.

Metric 2 — Total Δv

Δvtotal=ΔvA+ΔvB

Lower generally means less fuel expenditure.

Metric 3 — Negotiation latency

Tdecision

How long from threat detection to agreement?

Metric 4 — Communication overhead

Number of:

- messages,

- bytes,

- negotiation rounds.

Metric 5 — Computational cost

Measure:

- CPU time,

- memory,

- cryptographic operations.

Metric 6 — Robustness

Test:

- packet loss,

- delayed messages,

- malicious messages,

- satellite failure.

Metric 7 — Fairness

Measure whether one participant consistently bears the maneuver cost.

34. What Does “Fairness” Actually Mean?

This needs to be defined carefully.

Fairness does not necessarily mean:

Both satellites spend exactly the same amount of fuel.

That could actually be unfair.

Suppose:

A has 100 units fuel.

B has 5 units fuel.

Making both spend 2 units is very different.

A better concept might be:

Relative Costi=Maneuver CostiAvailable Maneuver Resourcei

Then we try to minimize the unfairness between the two parties.

This is an area where our project could develop an original decision metric.

35. Potential Algorithm

A first version could work like this:

1. Detect conjunction

2. Authenticate other satellite

3. Exchange minimum required data

4. Calculate collision probability

5. Generate candidate maneuvers

6. Calculate cost of each candidate

7. Calculate utility for each satellite

8. Generate preferred proposal

9. Exchange proposal

10. Evaluate proposal

11. Accept or counter-propose

12. Repeat until:

agreement

timeout

emergency

13. Cryptographically sign agreement

14. Execute maneuver

15. Recalculate trajectory

16. Verify collision risk

36. Where AI Could Fit

AI should not be the foundation of the project.

That would make the research less clear.

The core should be:

distributed protocol + security + optimization/game theory

AI can be added later.

For example, reinforcement learning could learn:

Which maneuver strategy tends to minimize long-term fuel cost?

But the final system should have deterministic safety constraints.

A good architecture would be:

AI / Optimization

↓

Candidate maneuver

↓

Safety verification

↓

Negotiation

↓

Execution

not:

AI

↓

"Trust me"

↓

Burn engine

37. The Biggest Technical Challenge

There is one particularly important issue we should not hide.

Orbital mechanics is not like two cars driving toward each other.

A tiny maneuver can change the spacecraft's future position substantially.

A maneuver that avoids the current collision could create:

a future collision.

Therefore, every proposed maneuver needs to be propagated forward.

Conceptually:

Statet+1=fStatetΔv

The system should therefore evaluate:

Current trajectory

↓

Candidate maneuver

↓

Future orbit propagation

↓

Collision probability

↓

Mission constraints

↓

Utility

This is why we cannot reduce the project to simply:

“Satellite with more fuel moves.”

38. A More Accurate Definition of Our Research

I would define the project as:

A decentralized software framework for secure autonomous negotiation between independently operated spacecraft during collision-risk events, combining inter-satellite communication, privacy-aware information exchange, multi-agent decision-making, game-theoretic maneuver selection, and safety verification.

That is considerably more precise than:

“Satellite collision avoidance.”

39. What We Are NOT Claiming

This is important for the research paper.

We should explicitly avoid claiming:

❌ “No autonomous collision avoidance exists.”

False.

Autonomous collision avoidance is already being actively researched and deployed.

❌ “Satellites cannot communicate directly.”

False.

Inter-satellite communication is established technology.

❌ “Ground stations are too slow because radio takes too long.”

That is an oversimplification.

The major issue is operational coordination, scale, intermittent information, and decision latency, not merely speed-of-light propagation.

❌ “No game theory has been applied to satellites.”

False.

Game-theoretic spacecraft control already exists.

❌ “Nobody has automated operator coordination.”

Also increasingly false. Industry systems such as Kayhan Space already provide automated/structured coordination mechanisms.

40. What We ARE Claiming

Our more defensible claim is:

Existing research has developed important components independently — autonomous avoidance, decentralized planning, secure collision-probability computation, inter-satellite communication, and operator coordination — but there remains an opportunity to investigate an integrated spacecraft-to-spacecraft negotiation protocol that combines these components into a lightweight, secure, distributed decision-making system.

That is a research claim we can actually defend.

41. The Project's Research Gap in One Diagram

EXISTING RESEARCH

│

┌───────────────┼────────────────┐

│               │                │

▼               ▼                ▼

Autonomous        Secure            Game

avoidance         computation       theory

│               │                │

└───────────────┼────────────────┘

│

▼

Decentralized

planning

│

▼

Communication

│

▼

┌─────────┐

│  GAP    │

│         │

│ Secure  │

│ +       │

│ Decentralized

│ +       │

│ Autonomous

│ +       │

│ Negotiation

│ +       │

│ Fairness│

└─────────┘

│

▼

OUR PROJECT

42. Our Proposed Contribution

The project could produce four concrete contributions.

Contribution 1 — Protocol

A machine-to-machine protocol for collision negotiation.

For example:

ASNP

Autonomous Satellite Negotiation Protocol.

Contribution 2 — Decision Algorithm

A game-theoretic/optimization algorithm for selecting:

- who moves,

- how much they move,

- when they move.

Contribution 3 — Security Layer

A lightweight security architecture providing:

- authentication,

- encryption,

- integrity,

- replay protection,

- signed agreements.

Contribution 4 — Simulation Framework

A testbed capable of measuring:

- collision risk,

- fuel usage,

- fairness,

- communication overhead,

- latency,

- computational cost,

- resilience.

43. The Research Question

The project can be centered around one main question:

How can independently operated autonomous satellites securely negotiate collision-avoidance maneuvers with minimal communication, computational overhead, and aggregate fuel consumption?

Then divide it into subquestions:

RQ1

How much trajectory information must satellites exchange to make a safe decision?

RQ2

Can cryptographic methods protect sensitive orbital information without making onboard negotiation too expensive?

RQ3

How should maneuver responsibility be allocated between two competing agents?

RQ4

Can game-theoretic negotiation reduce total maneuver cost compared with independent maneuvering?

RQ5

How should the protocol behave when communication fails?

RQ6

Can the protocol scale from two satellites to multiple simultaneous conjunctions?

44. Final Problem Statement

I would use this as the formal problem statement:

The rapid growth of LEO satellite constellations is increasing the number of conjunction events that require collision-risk assessment and maneuver decisions. Existing collision-avoidance systems rely heavily on ground-based tracking, operator-specific decision processes, and coordination mechanisms that are not designed for fully autonomous interaction between independently operated spacecraft. At the same time, research has separately explored autonomous maneuver planning, decentralized multi-spacecraft coordination, privacy-preserving collision-probability computation, inter-satellite communication, and game-theoretic control. However, an integrated lightweight protocol for secure spacecraft-to-spacecraft negotiation of collision-avoidance responsibility remains an open research direction.

This project proposes to investigate a decentralized software framework in which two autonomous spacecraft can authenticate one another, exchange the minimum information required to assess a conjunction, generate and evaluate maneuver options, negotiate maneuver responsibility according to safety, fuel, and mission-cost criteria, reach a cryptographically verifiable agreement, and safely execute or abandon the agreement under communication failures.

45. The Project in One Sentence

If someone asks you:

“What are you actually building?”

Don't say:

“A system where satellites communicate.”

Say:

“We are designing a secure distributed protocol in which two autonomous satellites can negotiate who should perform a collision-avoidance maneuver, using game theory and orbital constraints to minimize unnecessary fuel consumption while remaining safe under uncertain information and communication failures.”

That is the CSE project.

46. And the Core Idea in the Simplest Possible Form

The entire project can ultimately be understood as:

SATELLITE A

│

"We may collide."

│

▼

Secure handshake

│

▼

Share safe information

│

▼

Calculate possible moves

│

▼

┌─────────────┐

│ NEGOTIATION │

└─────────────┘

│

┌─────┴─────┐

▼           ▼

A moves      B moves

│           │

└─────┬─────┘

▼

Safe agreement

│

▼

Execute + verify

The spacecraft are the environment.

The orbital mechanics provides the constraints.

But the CSE contribution is the distributed intelligence, communication protocol, cybersecurity, negotiation algorithm, and fault-tolerant software architecture connecting the two spacecraft.

And that is the direction I would take the project.